| CS-461 Internetworking |  |  | Test #2 |
| --- | --- | --- | --- |
| Department of Computer Science |  |  | April 22, 2003 |
| James Madison University |  |  | Time Allowed: 60 Minutes |

| Name (PRINT) | SOLUTIONS |
| --- | --- |
| Score (Max 40 points) |  |

Attempt ALL questions. Put your answers in the provided space.

Identify and circle the correct answers for questions 1 through 6 (3 points each)

- In an OSPF autonomous system with n areas, how many areas are connected to the backbone area??

| - 1 | - n – 1 |
| --- | --- |
| - n | - n + 1 |

- A connectionless iterative server uses    ______    ports.

| - ephemeral | - well-known |
| --- | --- |
| - active | - a and b |

- In a connection-oriented concurrent server, the ______  is used for connection only.

| a. infinite port | b. ephemeral port |
| --- | --- |
| c. well-known port | d. b and c |

- The  _____  system call adds the local socket address to an already created socket.

  - address

  - create

  - bind

  - socket

- The  _____ system call creates a new socket for actual communication and connects the next client in line.

| a. accept | b. connect |
| --- | --- |
| c. bind | d. create |

- The  _____ system call establishes an active connection to a remote process.

| a. accept | b. bind |
| --- | --- |
| c. socket | d. connect |

- ( 8 points) A router has the following RIP routing Table:

| Destination | Hop Count |  | Next Hop |  |
| --- | --- | --- | --- | --- |
|  | Current | New | Current | New |
| Net_1 | 4 | 3 | B | C |
| Net_2 | 2 | 5 | C | C |
| Net_3 | 1 | 1 | F | F |
| Net_4 | 5 | 5 | G | G |

What would be the content of the above table if the router receives the following RIP response message from router C:

Indicate your answer by filling the new “Hop Count” and “Next Hop” entries in the above table.

| Destination | Hop Count |
| --- | --- |
| Net_1 | 2 |
| Net_2 | 4 |
| Net_3 | 3 |
| Net_4 | 7 |

- ( 4 points) During an FTP session, two connections are established; these are

The   Control   connection, which is used for exchanging commands and responses

and the  Data   connection, which is used for  transferring data files

- An IP datagram with no options carries a payload data of 4500 bytes and has an identification number of  36,952. The packet is to be fragmented into three smaller fragments of sizes: 1600, 1600, and 1300 bytes, respectively. For each of the three fragments, provide some of the headers’ fields by filling the following table:

- ( 4 points) What are the source and destination ports shown in an SNMP response message from the agent to the manager?

Source port is:		well-known port 161

Destination port is:	ephemeral port

- ( 6 points) Decode the following BER-encoded data (show values in decimal format)

      - 0A 40 04 23 51 62 71 02 02 14 12

| The sequence: Type | Tag (Hex) | Type | Tag (Hex) |
| --- | --- | --- | --- |
| INTEGER | 02 | IPAddress | 40 |
| OCTET STRING | 04 | Counter | 41 |
| OBJECT IDENTIFIER | 06 | Gauge | 42 |
| NULL | 05 | TimeTicks | 43 |
| Sequence, sequence of | 30 | Opaque | 44 |

IP address  35.81.98.113

- INTEGER  5138