Review Questions on
“WIntel” Computer
Hardware

© 1999 Charles Abzug

- What is the difference between the earlier Interrupt Address Table and the (current) Interrupt Descriptor Table?

- Calculate the bandwidth of each of the following buses:

| Bus Name | Cycle Rate (MHz) | Width (bits) | Bandwidth (MB/sec) |
| --- | --- | --- | --- |
| Original PC | 4.77 | 8 |  |
| PC/AT (ISA) | 8.33 | 8 or 16 |  |
| Micro Channel Architecture (MCA) | 20 or 40 | 16 or 32 |  |
| EISA | 8.33 | 32 |  |
| VESA Local (VL) | 33.33 | 32 |  |
| PCI (early version) | 33.33 | 32 |  |
| PCI (later version) | 66.66 | 64 |  |
| Accelerated Graphics Port (AGP) | 133.33 | 64 |  |

- What operations are performed on a floppy disk by the formatting program?

- What additional operations are carried out if the floppy disk is formatted as Bootable?

- What are the principal performance areas that can lead to floppy drive malfunction?

- Calculate the capacity of each of the following hard disk drives:

| Number of Tracks | Number of Heads | Sectors/ Track | Bytes | Mbytes | Gbytes |
| --- | --- | --- | --- | --- | --- |
| 855 | 7 | 17 |  |  |  |
| 1,024 | 5 | 26 |  |  |  |
| 900 | 15 | 17 |  |  |  |
| 1,023 | 15 | 17 |  |  |  |
| 4,096 | 18 | 72 |  |  |  |

- What is zone bit recording?

- What are the three modes in which BIOS can support a hard drive?

- What are the three principal commands of the DEBUG utility, and what does each of them do?

- For each disk drive, calculate the number of clusters and the size of the FAT.

| Number of Tracks | Number of Heads | Sectors/ Track | Bytes | Cluster Size | # of Clusters | Sectors in FAT |
| --- | --- | --- | --- | --- | --- | --- |
| 855 | 7 | 17 | 52,093,440 | 2,048 |  |  |
| 1,024 | 5 | 26 | 68,157,440 | 2,048 |  |  |
| 900 | 15 | 34 | 235,007,968 | 4,096 |  |  |
| 2,048 | 15 | 34 | 1,069,567,520 | 16,384 |  |  |
| 4,096 | 15 | 64 | 2,013,265,920 | 32,768 |  |  |

# Answers to Selected Questions:

- What is the difference between the earlier Interrupt Address Table and the (current) Interrupt Descriptor Table?

Answer:  See Andrews page 122.

- Calculate the bandwidth of each of the following buses:

Answer:

| Bus Name | Cycle Rate (MHz) | Width (bits) | Bandwidth (MB/sec) |
| --- | --- | --- | --- |
| Original PC | 4.77 | 8 | 2.39 |
| PC/AT (ISA) | 8.33 | 8 or 16 | 8.33 or 16.66 |
| Micro Channel Architecture (MCA) | 20 or 40 | 16 or 32 |  |
| EISA | 8.33 | 32 | 33.33 |
| VESA Local (VL) | 33.33 | 32 | 133.33 |
| PCI (early version) | 33.33 | 32 | 133.33 |
| PCI (later version) | 66.66 | 64 | 533.33 |
| Accelerated Graphics Port (AGP) | 133.33 | 64 | 1,066.66 |

- What operations are performed on a floppy disk by the formatting program?

Answer:

Delimits and marks Tracks and Sectors (all written with F6h).

Creates the master boot record.

Creates the File Allocation Table (FAT)    two copies.

Creates the root directory.

- What additional operations are carried out if the floppy disk is formatted as Bootable?

Answer:

For DOS/Windows3.1/3.11/95/98, three files are also copied to the disk:

MSDOS.SYS

IO.SYS

COMMAND.COM

- What are the principal performance areas that can lead to floppy drive malfunction?

Answer:

Azimuth skew

Hub Centering

Hysteresis

Radial Alignment

Rotational Speed

Sensitivity

- Calculate the capacity of each of the following hard disk drives:

Answer:

| Number of Tracks | Number of Heads | Sectors/ Track | Bytes | Mbytes | Gbytes |
| --- | --- | --- | --- | --- | --- |
| 855 | 7 | 17 | 52,093,440 | 49.68 |  |
| 1,024 | 5 | 26 | 68,157,440 | 65 |  |
| 900 | 15 | 17 | 117,503,984 | 112.06 |  |
| 1,023 | 15 | 17 | 133,562,868 | 127.4 |  |
| 4,096 | 18 | 72 | 2,717,908,992 | 2,592 | 2.53 |

- What is zone bit recording?

Answer:  See Andrews, page 198.

- What are the three modes in which BIOS can support a hard drive?

Answer:  CHS or Normal Mode, Large Mode, and LBA (Logical Block Address) Mode

- What are the three principal commands of the DEBUG utility, and what does each of them do?

Answer:  Load, Dump, Enter, and Write.  For explanation of their usage, see Andrews’ Table 5-1 on page 203.

- For each disk drive, calculate the number of clusters and the size of the FAT.

Answer:

| Number of Tracks | Number of Heads | Sectors/ Track | Bytes | Cluster Size | # of Clusters | Sectors in FAT |
| --- | --- | --- | --- | --- | --- | --- |
| 855 | 7 | 17 | 52,093,440 | 2,048 | 25,437* | 100 |
| 1,024 | 5 | 26 | 68,157,440 | 2,048 | 33,280 | 130 |
| 900 | 15 | 34 | 235,007,968 | 4,096 | 57,375 | 225 |
| 2,048 | 15 | 34 | 1,069,567,520 | 16,384 | 32,640 | 128 |
| 4,096 | 15 | 64 | 2,013,265,920 | 32,768 | 61,440 | 240 |