Homework # 13

Name ___________________

Finish the problem we started in class (taken from  Table 2 on page 753  and Figure 2 on page 753)  by adding the excitations that would be necessary if we were to implement this machine (sequential circuit) with either  JK, D,  or T flip-flops. The table below represents where we were at the end of the class.

NOTE: our state diagram does not correspond to FIGURE 2 because we are treating f and g as separate functions.   Our function f  just has state changes (no output).   The book has a misprint in Table 2 because they label g as input  when the text states that g represents output.

| current | state | input | next | state | SA | RA | SB | RB | DA | DB | TA | TB | JA | KA | JB | KB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | x | A | B |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 1 | 0 | X | 1 | 0 | 0 | 1 | 0 | 1 | 0 | X | 1 | X |
| 0 | 0 | 1 | 0 | 0 | 0 | X | 0 | X | 0 | 0 | 0 | 0 | 0 | X | 0 | X |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | X | 0 | 1 | 1 | 1 | 0 | 1 | X | X | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | X | 0 | 1 | 0 | 0 | 0 | 1 | 0 | X | X | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | X | 1 | 1 | X |
| 1 | 0 | 1 | 1 | 0 | X | 0 | 0 | X | 1 | 0 | 0 | 0 | X | 0 | 0 | X |
| 1 | 1 | 0 | 1 | 0 | X | 0 | 0 | 1 | 1 | 0 | 0 | 1 | X | 0 | X | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 | 1 | X | 0 | 0 | 1 | 1 | 0 | X | 1 | X | 0 |