Homework #7  KEY

Section 2.6 page 180 in textbook

- #30 part a  Find the one’s complement representations, using bit strings of length six, of  2210

0101102

- #30 part c  Find the one’s complement representations, using bit strings of length six, of  -710

+7 =   0001112         -7 = 1110002

- #36  part a Find the two’s complement representations, using bit strings of length six, of  2210

0101102

remember:  positive numbers are the same in all three representations

- #36 Part c  Find the two’s complement representations, using bit strings of length six, of  -710

-710 = 1110012

The next 6 questions relate to the Boolean function described below by a Karnaugh map:

|  | cd |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ab | \ | 00 | 01 | 11 | 10 |
|  | 00 | 1 |  |  | 1 |
|  | 01 | 1 | x |  | 1 |
|  | 11 | x |  | 1 |  |
|  | 10 | 1 |  | 1 |  |

- find its simplest Product of Sums expression

|  | cd |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ab | \ | 00 | 01 | 11 | 10 |
|  | 00 | 0 | 1 | 1 | 0 |
|  | 01 | 0 | x | 1 | 0 |
|  | 11 | x | 1 | 0 | 1 |
|  | 10 | 0 | 1 | 0 | 1 |

F’(a,b,c,d) = a’d +  c’d + acd’

F’’(a,b,c,d) = F(a,b,c,d) = (a’d +  c’d + acd’)’ =  (a’’+d’)(c’’+d’)(a’+c’+d’’) =  (a+d’)(c+d’)(a’+c’+d)

- find its equivalent  Sum of Products expression

|  | cd |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ab | \ | 00 | 01 | 11 | 10 |
|  | 00 | 1 |  |  | 1 |
|  | 01 | 1 | x |  | 1 |
|  | 11 | x |  | 1 |  |
|  | 10 | 1 |  | 1 |  |

F(a,b,c,d) =    c’d’ +  a’d’ + acd

- Using truth tables, show that your answers to 5 and 6 produce the same function

| a | b | c | d | a' | c' | d' | c'd' | a'd' | acd | SOP | a+d' | c+d' | a'+c'+d | POS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |

- Write the answer to #56   in disjunctive normal form –

It is actually already in disjunctive normal form:  c’d’ +  a’d’ + acd

If you expand it so that each minterm  contains each variable,

this is what you get (minus the green minterms which are duplicates of the minterms below them)

a’d’ = a’bd’ + a’b’d’ =  a’bcd’ + a’bc’d’ + a’b’cd’ + a’b’c’d’ +

c’d’ = ac’d’ +  a’c’d’ =  abc’d’ + ab’c’d’ + a’bc’d’ + a’b’c’d’ +

acd = abcd + ab’cd

- Write the answer to #65   in conjunctive normal form

It is actually already in conjunctive normal form:    (a+d’)(c+d’)(a’+c’+d)

If you expand it so that each maxterm contains each variable,

this is what you get:

(a+b+c+d’)(a+b+c’+d’)(a+b’+c+d’)(a+b’+c’+d’)(a’+b’+c+d’)(a’+b+c+d’)(a+b’+c+d’)(a+b+c+d’)(a’+b’+c’+d)(a’+b+c’+d)

Note that the underlined maxterms are duplicates  as are the outlined maxterms

- Using MMLogic (or any other circuit design program)  - construct the circuit for either #5  or #6 (inputs should be switches, output should be a lamp).  Bring a disk with your circuit on it to class (as well as a printout).