Sequential Circuits & Flip-Flops

RS flip-flop

| S | R | Q(t+1) | Comments |
| --- | --- | --- | --- |
| 0 | 0 | Q(t) | no change |
| 0 | 1 | 0 | clear |
| 1 | 0 | 1 | set |
| 1 | 1 | ? | not allowed |

SR Characteristic Table

D or data flip-flop is a modification of the RS flip-flop

(inverter is inserted between S and R and send in D to the S input)

| D | Q(t+1) | Comments |
| --- | --- | --- |
| 0 | 0 | Clear |
| 1 | 1 | Set |

D Characteristic table

JK flip-flop is a refinement of the RS flip-flop

in that the indeterminate condition of the RS flip-flop is defined in the JK type

| J | K | Q(t+1) | Comments |
| --- | --- | --- | --- |
| 0 | 0 | Q(t) | no change |
| 0 | 1 | 0 | clear |
| 1 | 0 | 1 | set |
| 1 | 1 | Q’(t) | complement |

JK Characteristic Table

T flip-flop is a refinement of the JK flip-flop

in that the inputs J and K are tied together.

| T | Q(t+1) | Comments |
| --- | --- | --- |
| 0 | Q(t) | Clear |
| 1 | Q’(t) | Set |

T Characteristic table

Excitation Tables for the Four types of Flip-Flops

SR flip-flop

| Q(t) | Q(t+1) | S | R |
| --- | --- | --- | --- |
| 0 | 0 | 0 | X |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | X | 0 |

JK flip-flop

| Q(t) | Q(t+1) | J | K |
| --- | --- | --- | --- |
| 0 | 0 |  |  |
| 0 | 1 |  |  |
| 1 | 0 |  |  |
| 1 | 1 |  |  |

D flip-flop

| Q(t) | Q(t+1) | D |
| --- | --- | --- |

T flip-flop

| Q(t) | Q(t+1) | T |
| --- | --- | --- |

Excitation Tables for the Four types of Flip-Flops

RS flip-flop

| Q(t) | Q(t+1) | S | R |
| --- | --- | --- | --- |
| 0 | 0 | 0 0 0 | 0 1 X |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 0 X | 0 0 0 |

JK flip-flop

| Q(t) | Q(t+1) | J | K |
| --- | --- | --- | --- |
| 0 | 0 | 0 0 0 | 0 1 X |
| 0 | 1 | 1 1 1 | 0 1 X |
| 1 | 0 | 0 1 X | 1 1 1 |
| 1 | 1 | 0 1 X | 0 0 0 |

D flip-flop

| Q(t) | Q(t+1) | D |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

T flip-flop

| Q(t) | Q(t+1) | T |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

The rest of the material was on handouts.